Ros Notes:

Aim: The Raspberry Pi captures images and generates structured JSON data, which is transmitted using HTTP POST requests to a Flask-based server. The server receives and processes the data, demonstrating communication between system components.

The system captures images, creates structured data, sends it via HTTP to a Flask server, and displays the result on a dashboard

Camera → JSON → HTTP → Flask → Dashboard

Before sending the proposal – trial to be sure I can get everything working again.

How to complete this:

My VM crashed. – create a new one

I created a new VM, accessed my raspberry pi through

ssh roisin@

I had to set up git hub on my raspberry. Use co-pilot for this.

Sudo apt update

Sudo apt install git -y

And then I had to configure it.

git config --global user.name "RoisinRussell2025"

git config --global user.email roisin_russell@yahoo.ie

I had issues with moving networks as I have home and office.

Work around was to plug pi directly into the office network.

A lot of troubleshooting for the original git commit but now its done I will use the code below to update regularly. Set up the automatic token recall so I don’t have to enter credentials all the time.

git add .

git commit -m "message on what I do"

git push

To start my project  I will need to create new directories

1 – my project name

Mkdir smart-camera-monitor

Into the file to create the next folders I need..

Cd – smart-camera-monitor

mkdir static

mkdir data

mkdir docs

nano camera_capture.py

Static files = files that don’t change dynamically i.e. images, For this project the camera will save image and the flask dashboard will show it.

Data = this will record my timestamps, my event logs and system information. (info for Json)

Docs – is my assignment workings.

Next step is my input  - make sure the camera works.

I needed to add the camera again because of all the crashing.

“I used the rpicam-jpeg tool provided by Raspberry Pi to capture images from the camera module using the modern camera stack.”

sudo apt update

sudo apt install libcamera-apps -y

I then needed to run

Rpicam-jpeg -o test.jpg to test it.

scp roisin@192.xxx:~/test.jpg

both of these confirm that there is now a test.jpg saved and that the camera is working.

This is the end of week2 and input layer (camera) is working

Next is to create the python script. To send the data.

my camera_capture.py

I used the script from the lab and adjusted it to reflect my own project.

The lab had

from sense_hat import SenseHat

sense = SenseHat()

for event in sense.stick.get_events():

if event.action == "pressed" and event.direction == "middle":

I will replace this with:

an automatic timed trigger.

I need to add areas to create json and get it ready for the next step.

These included “import Json”

Creating an event - I looked up how “python datetime format string”

data = {

"timestamp": "...",

"event": "image_captured"

}

to save a json file and get it ready for networking.

Full python code used:

from picamera2 import Picamera2

import time

import json

from datetime import datetime

# Start camera

picam2 = Picamera2()

picam2.start()

while True:

# Capture image

image_path = "static/last_capture.jpg"

picam2.capture_file(image_path)

# Create event data

data = {

"timestamp": datetime.now().strftime("%Y-%m-%d %H:%M:%S"),

"event": "image_captured"

}

# Save JSON data

with open("data/data.json", "w") as f:

json.dump(data, f)

# Print output – An f string is a way to embed expression./variables directly within strings

print(f"Captured image at {data['timestamp']}")

# Wait 10 seconds

time.sleep(10)

Next, I want to create the python script to receive the data. Server.py

it will need to act like a server, wait for requests, process data and send responses.

In lab 9 – we imported flask and flask cor – Flask-CORS was not required as communication was carried out between Python, rather than a browser-based frontend, so no cross-origin restrictions applied.”

I am going to import flask (new vm, reconfigured pi (100 times))

I am going to do this through a server.py file.

The Raspberry Pi sends structured JSON data via HTTP POST requests to a Flask-based server, demonstrating application layer communication between system components

I will be replacing the Get in the lab 9 with POST – this is because I am not looking to get data but I am looking to send/post data. The Flask route listens for POST requests, extracts the JSON using request.json, and processes or logs the received data

import json

import os

from flask import Flask, request, render_template

BASE_DIR = os.path.dirname(os.path.abspath(__file__))

DATA_DIR = os.path.join(BASE_DIR, "data")

os.makedirs(DATA_DIR, exist_ok=True)

DATA_PATH = os.path.join(DATA_DIR, "data.json")

app = Flask(__name__, static_folder="static", template_folder="templates")

@app.route("/data", methods=["POST"])

def receive_data():

data = request.json

# Save latest event to file (so dashboard can read it)

with open(DATA_PATH, "w") as f:

json.dump(data, f)

print("Received data:", data)

return {"status": "ok"}

def load_latest_event():

try:

with open(DATA_PATH, "r") as f:

return json.load(f)

except FileNotFoundError:

return None

except Exception as e:

print("Error reading data.json:", e)

return None

@app.route("/", methods=["GET"])

def dashboard():

last_event = load_latest_event()

return render_template("status.html", last_event=last_event)

Creating the dashboard.

1 – I will create a new directory called templates. I added in my status html to this so Flask can find it – and know what the look should be.

The dashboard was implemented using Flask templates and based on a lab design, but adapted to display camera event data and images rather than environmental sensor readings. I will need assistance with this one as I don’t have the web dev done.